INBG 417

Exam Assignment 1

12 June 2026

Nthabiseng Maoka

30224926

Submitted in pursuit of the degree

BACHELOR OF ENGINEERING

In

INDUSTRIAL ENGINEERING

North-West University

Potchefstroom Campus

North-West University: Potchefstroom Campu

Table of Contents

List of Tables	0

List of Figures	0

List of Equation	0

1.	Project Background	0

2.	Design Problem, Aim and Objectives	0

2.1.	Aim	0

2.2.	Objectives	0

3.	Design Requirements, Assumptions and Constraints	1

4.	Design requirements summary	1

5.	Key assumptions	1

6.	Design Constraints and Implications	2

7.	Product and process understanding	0

1.1	Product structure	0

Executive summary

This report presents

DigMore Co., an established shovel manufacturer, is extending its range with an all-purpose folding mini shovel for the camping market, and because its existing plant is already committed to current lines and cannot absorb the new demand, a dedicated facility is required; this report applies a systematic engineering design approach to develop that facility's layout and Master Facility Plan (MFP). The design is sized for a projected demand of 400,000 shovels per year, roughly 1,819 per day on a single nine-hour shift across 220 working days (derived in Section 6) — and the product is built from three aluminium parts: a back handle section, a middle handle section, and a shovel-head assembly of a stamped head and hinge connector riveted together, with a 5% scrap allowance carried at tube cutting and sheet stamping. All materials are stored on site, covering two weeks of raw material and four weeks of finished goods; the powder-coating area is fully enclosed to contain fumes; the site provides public-road access with weekday receiving, on-site collection of small orders and loading space for DigMore's own delivery fleet; and about 10% of the floor area is reserved for projected growth over the next five years. Three alternative block layouts were assessed against [VERIFY: evaluation criteria], and the selected option, Alternative [VERIFY], was refined into a final MFP of approximately [VERIFY: total area] m² covering receiving, storage, production, the enclosed coating area, dispatch, offices and personnel facilities, giving management a defendable basis for establishing the new facility.

### List of Tables

Table 1: folding mini shovel raw material components	0

### List of Figures

Figure 1: Folding Mini Shovel Concept Design	0

### List of Equation

# Executive Summary

# Academic Integrity and AI Declaration

# Project Background

DigMore Co. is an established South African shovel manufacturer with a broad customer base across the camping and outdoor market. The company has decided to expand its product range with a new all-purpose folding mini shovel. This compact product consists of two aluminium handle sections (back and middle) manufactured from round tube and a shovel-head assembly produced from aluminium sheet.

The projected annual demand for the folding mini shovel is 400 000 units. DigMore Co.’s existing production facility is already fully utilised by current product lines and cannot absorb this additional volume. Management has therefore decided to establish a new dedicated manufacturing facility for the folding mini shovel only.

An Industrial Engineer has been engaged to develop a comprehensive facility design that meets the projected demand while addressing receiving, storage, production, work-in-progress, finished-goods storage, customer collection and dispatch requirements. The design must also incorporate a fully enclosed powder-coating area for safety and reserve approximately 10% of the floor area for future expansion. This report presents the systematic engineering design process followed, including capacity analysis, process requirements, space planning, alternative layout development and the final Master Facility Plan.

# Design Aim and Objectives

## 2.1. Aim

Design a dedicated manufacturing facility for DigMore Co. that meets the projected demand, of 400 000 folding mini shovels per year, provides efficient receiving and dispatch operations, ensures safe and logical material flow, and allows for future expansion.

## 2.2. Objectives

The following design objectives have been established to guide the development of the new facility:

Production and Capacity

To design a manufacturing facility capable of meeting the annual demand of 400 000 folding mini shovels, incorporating the required production capacity, scrap allowance and future expansion

Site and Access

Recommend a site layout that provides safe access from public roads for supplier deliveries, customer collections, employee access and DigMore Co.’s own delivery fleet.

Layout and Material Flow:

Develop a layout that supports a clear, logical flow of materials from receiving through raw-material storage, production processes, work-in-progress staging, assembly, packaging and finished-goods storage to dispatch.

Equipment and Workstations:

Select appropriate machinery, equipment and workstation designs for all required operations, including cutting, threading, stamping, bending, sharpening, serrating, riveting, heat treatment, sandblasting, polishing, powder coating, assembly and packaging.

Storage and Material handling:

Provide sufficient indoor storage for two weeks of raw materials, four weeks of finished goods and appropriate work-in-progress areas between production stages.

Safety and Support Facilities:

Incorporate all safety requirements, including a fully enclosed powder-coating area, and provide suitable offices, amenities, parking, utilities and other support facilities for management, administrative staff and operators.

# Design Requirements

The following design requirements, categorised under their relevant headings, have been established to guide the successful design of DigMore Co.’s dedicated folding mini shovel manufacturing facility.

Production requirements

The facility shall:

- Produce 400 000 folding mini shovels per year.

- Provide sufficient production capacity for one 9-hour shift per day, five days per week, based on 220 working days per year.

- Account for operator breaks when determining available productive time per shift.

- Include a 5% scrap allowance at the tube cutting and sheet stamping processes only.

- Provide manual packaging workstations, as packaging also serves as the final visual inspection point before shipping.

Figure 1:Folding mini shovel concept design

Manufacturing Process requirements

The facility shall:

- Accommodate the complete manufacturing process for the handle cutting, internal threading, grip forming, knurling, external threading, hinge connector stamping, hinge connector bending, shovel head stamping, sharpening, serrating, shovel head bending, riveting, heat treatment, sandblasting, polishing, powder coating, assembly and packaging.

- Provide suitable machines, workstations and operator space for each operation.

- Include work-in-progress staging points between operations where required by the production design.

- Provide process flow charts to support the quality management system and to demonstrate the sequence of manufacturing operations.

Site and access requirements

The facility shall:

- Be accessible from public roads for supplier deliveries, customer collections, employee access and delivery trucks.

- Provide sufficient space for raw-material delivery vehicles to enter, offload and exit the site safely.

- Provide sufficient space for finished-goods dispatch vehicles and company delivery trucks.

- Include a designated collection point for small customer orders.

- Provide space on the premises for the truck fleet.

- Include suitable parking, security control and access control for employees, customers, visitors and goods vehicles.

- Allocate approximately 10% of the floor area for future expansion over the next five years.

Figure 3:Site access and vehicle movement layout

Layout and Flow requirements

The facility shall:

- Provide a logical flow of materials from receiving to raw-material storage, production, work-in-progress staging, assembly, packaging, finished-goods storage and dispatch.

- Provide clear movement paths for operators, materials and handling equipment between departments.

- Reduce unnecessary backtracking between raw materials, work-in-progress, finished goods and dispatch areas.

- Position finished-goods storage close to dispatch to limit unnecessary movement of completed products.

- Position packaging after assembly and before finished-goods storage to support the final visual check before shipping.

Storage and Material Handling

The facility shall:

- Store all raw materials, components, work-in-progress and finished goods inside the building.

- Provide raw-material storage for two weeks of production stock, including aluminium round tubes, aluminium sheets or coils, and bought-in packaging materials.

- Provide finished-goods storage for four weeks of completed folding mini shovels.

- Provide work-in-progress storage between operations where required by the selected production system.

- Include staging areas for materials and components at relevant manufacturing and assembly workstations.

- Use material-handling methods and aisle widths suited to the selected unit loads, storage arrangements and movement volumes.

[Insert Figure 3.4 here: High-level material flow requirement from receiving to dispatch.]

Figure 3.4: High-level material flow requirement from receiving to dispatch

Receiving and Dispatch

The facility shall:

- Include a designated receiving area for supplier deliveries during weekday working hours.

- Provide space for offloading, inspection and transfer of raw materials to storage or production.

- Include a dispatch area for large orders transported by DigMore Co.’s own delivery trucks.

- Provide a separate or clearly controlled collection point for small customer orders.

- Include staging space for completed orders before loading or customer collection.

Figure 4: truck dimensions

Figure 5: loading bay

Ensure that receiving and dispatch activities do not obstruct production movement or operator pathways.

Safety and Waste Management

The facility shall:

- Locate the powder-coating process in a separate, fully enclosed area within the building due to fume-related safety concerns.

- Provide safe separation where required for cutting, stamping, heat treatment, sandblasting, sharpening, serrating and powder-coating operations.

- Provide clear operator pathways around workstations and material-handling routes.

- Include waste and scrap handling areas, especially for scrap generated during tube cutting and sheet stamping.

- Position scrap and waste collection points so that waste can be removed without disrupting production flow.

Supporting Facilities

The facility shall:

- Provide office space for the Senior Plant Manager, Operations Manager, Inventory Manager, administrative staff, reception, purchasing, sales, finance, marketing, human resources and plant supervisors.

- Provide personnel facilities, including restrooms, lockers and cafeteria access for all employees.

- Provide parking for management, administrative staff, security personnel, operators, customers and company vehicles.

- Include utility requirements such as water, electricity, plumbing, lighting and ventilation.

- Include security facilities for controlling the movement of people, materials and vehicles on site.

Figure 6: parking spaces

Key Layout Implications for Development

The requirements above give rise to the following key implications that will be used when developing and evaluating alternative block layouts:

- Raw-material storage should be positioned close to the first production operations.

- Work-in-progress staging areas should be positioned between related operations without creating excess inventory.

- The enclosed powder-coating area should be separated from general production movement, while still allowing practical entry and exit of parts.

- Packaging must be located after assembly and before finished-goods storage.

- Finished-goods storage should be positioned close to dispatch.

- Receiving, customer collection and dispatch should be arranged so that external vehicle movement does not interfere with internal production flow.

- The 10% future expansion area should be clearly visible on the final Master Facility Plan and should not be used to compensate for current insufficient capacity.

# Assumptions and Constraints

This section summarises the key assumptions and constraints that underpin the facility design.

## Assumptions

Table 1: Key assumptions used in the facility design

| Assumption | Details |
| --- | --- |
| Annual demand | The facility is designed to produve 400 000 folding mini shovels per year. |
| Operating schedule | One 9-hour shift per day, five days per week, for 220 working days per year. |
| Productive time per shift | Operators receive one 30-minute lunch break and two 15-minute tea breaks. This gives 480 minutes of productive time per shift before allowances for set-up, maintenance or cleaning |
| Overtime | Overtime is available only as a flexible capacity measure and is not included in the base design. |
| Raw-material inventory | Raw-material storage is sized for two weeks of production requirements. |
| Finished-goods inventory | Finished-goods storage is sized for four weeks of completed shovels |
| Work-in-progress inventory | WIP storage quantities and locations are determined by the chosen production system and are provided only where staging is required between operations. |

## Constraints

The final facility design must satisfy the following constraints:

- The facility must be dedicated to the folding mini shovel product line.

- A 5% scrap allowance applies only to the tube cutting and sheet stamping processes.

- All raw materials, components, work-in-progress and finished goods must be stored inside the facility.

- The powder coating process must be in a separate, fully enclosed area within the building.

- All supplier deliveries, customer collections and finished-goods dispatch activities must occur during weekday working hours.

- Adequate space must be reserved on the premises for DigMore Co.’s own delivery truck fleet.

- Approximately 10% of the facility floor area must be reserved for future expansion within the next five years.

# Process Flow

The facility layout is developed based on the manufacturing process sequence of the folding mini shovel, ensuring that materials move in a structured and continuous manner through the system.

The process begins with the receiving and storage of raw materials, including aluminium tubes and sheets. These materials are then introduced into the production process, where initial cutting and forming operations are performed.

Subsequently, handle components undergo threading and knurling operations, while sheet metal components are stamped and shaped to form shovel heads. The shovel head components are further processed through sharpening, serrating, and bending operations before being joined to hinge connectors.

Following fabrication, components undergo heat treatment and surface preparation processes, including sandblasting and polishing. Powder coating is then applied to provide the required surface finish, after which final assembly is completed.

The finished product is packaged and transferred to finished goods storage, from where it is either collected by customers or dispatched via company transport.

Figure 2: Process Flow for Folding Mini Shovel Manufacturing

This process flow forms the foundation for workstation definition, departmental grouping, and the development of alternative facility layouts.

# Detailed Design Specifications

This section presents the detailed design specifications used to develop the proposed manufacturing facility for DigMore Co. The purpose of this section is to translate the design brief into measurable production, raw material, process, and capacity requirements. The calculations in this section form the basis for later workstation planning, space allocation, department sizing, and alternative layout development.

## Demand

DEMAND

## Raw Material Requirements

### 6.1.1 Raw material Overview

The raw materials supplied by the supplier are delivered in standard stock forms, primarily in batches of 6 m lengths for tubular materials and sheet/coil form for flat materials. The current supplier for both material types is Aluminium Trading,

- Aluminium round tubes (31.70 mm outer diameter × 3.18 mm wall thickness × 6000 mm) are used to manufacture the shovel handle sections (back and middle sections).

- Rolled aluminium sheets/coils (2.5 mm thickness) are used to manufacture the shovel head and hinge connector through stamping and forming processes.

|  | Processing Time |
| --- | --- |
| Cut handle | 15 s per cut (number of pipes per cut depends on equipment) |
| Internal thread on the middle section | 20 s per pipe |
| Stamp form end of grip section (narrowing) | 5 s per pipe |
| Knurling and external thread on grip section | 1 min per pipe |
| Stamp hinge connector | 2 s per part |
| Bend hinge connector | 5 s per part |
| Stamp shovel head | 2 s per part |
| Sharpen shovel head blade | 2 min per shovel |
| Serrate shovel head blade edge | 1 min per shovel |
| Shape/bend shovel head | 2 s per shovel head |
| Rivet hinge connector to shovel head | 5 s per shovel |
| Heat treat shovel head subassembly | Depends on equipment |
| Sandblasting shovel heads | Depends on equipment |
| Hand polish to remove burrs | 1 min per shovel head |
| Powder-coat all parts (black colour) | Depends on equipment |
| Assemble shovel (handle + head) | 30 s per shovel |
| Packaging (bagging and boxing) | Depends on workstation design |

| Table 6.2: Raw Material Item, Dimensions and Use |  |  |
| --- | --- | --- |
| Raw material | Dimensions / specification | Use |
| Aluminium round tube | 31.70 mm outside diameter, 3.18 mm wall thickness | Back handle and middle handle sections |
| Aluminium sheet | 2.5 mm thickness | Shovel head and hinge connector |
| Rivets | Assumed standard aluminium/steel rivets | Joining hinge connector to shovel head |
| Packaging bags and boxes | Assumed bought-in packaging material | Final packaging |

The tube and sheet specifications are taken from the assignment brief. Rivet and packaging details are not fully specified in the brief and are therefore treated as design assumptions.

### 6.1.3 Finished Goods Demand

The required annual demand is 400 000 folding mini shovels per year. The company operates for 220 workdays per year.

The daily production demand is calculated from the annual demand and the number of working days per year, as shown in Equation 6.1.

Equation 6.1: Daily production demand
Dday = 400 000 / 220
Dday = 1 818.18 ≈ 1 819 shovels/day

The productive working time per day is calculated from the 9-hour shift less a 30-minute lunch break and two 15-minute tea breaks. This gives 8 productive hours per day.

Using 8 productive hours per day, the hourly production demand is determined as shown in Equation 6.2.

Equation 6.2: Hourly production demand
Dhour = 1 818.18 / 8
Dhour = 227.27 ≈ 228 shovels/hour

Table 6.3: Finished Goods Demand Requirements

| Product | Annual demand | Working days/year | Daily demand | Hourly demand |
| --- | --- | --- | --- | --- |
| Folding mini shovel | 400 000 | 220 | 1 819 | 228 |

The table converts the annual market demand into daily and hourly production requirements using the operating calendar provided in the brief.

### 6.1.4 Component Demand

Each finished shovel requires one back handle section, one middle handle section, one shovel head, and one hinge connector. Therefore, the daily component demand before scrap is 1 819 units per component.

Each finished shovel requires one back handle section and one middle handle section. Therefore, the total daily tube-component demand before scrap is calculated as shown in Equation 6.3. A 5% scrap allowance is then applied to determine the required input quantity.

Equation 6.3: Daily tube-component demand before scrap
Qtube,before = 1 819 + 1 819
Qtube,before = 3 638 components/day

Equation 6.4: Daily tube-component demand after scrap
Qtube,after = 3 638 / (1 - 0.05)
Qtube,after = 3 829.47 ≈ 3 830 components/day

For the stamped sheet-metal components, the required input quantity is calculated using the same scrap-adjustment relationship, as shown in Equations 6.5 and 6.6.

Equation 6.5: Daily shovel-head demand after scrap
Qhead,after = 1 819 / (1 - 0.05)
Qhead,after = 1 914.74 ≈ 1 915 heads/day

Equation 6.6: Daily hinge-connector demand after scrap
Qhinge,after = 1 819 / (1 - 0.05)
Qhinge,after = 1 914.74 ≈ 1 915 connectors/day

Table 6.4: Daily Component Demand

| Component | Components per shovel | Daily demand before scrap | Daily demand after scrap |
| --- | --- | --- | --- |
| Back handle section | 1 | 1 819 | Included in tube total |
| Middle handle section | 1 | 1 819 | Included in tube total |
| Total tube components | 2 | 3 638 | 3 830 |
| Shovel head | 1 | 1 819 | 1 915 |
| Hinge connector | 1 | 1 819 | 1 915 |

The scrap rate is applied only to the processes specified in the assignment brief.

### 6.1.5 Raw Material Inventory Requirement

The company’s raw material inventory policy requires two weeks’ worth of raw materials to be stored due to long supplier lead times. Since the company operates five days per week, this corresponds to 10 working days of raw material stock.

The raw-material inventory policy requires two weeks of stock. Since the facility operates five days per week, this corresponds to 10 working days of inventory. The corresponding stock levels are calculated in Equations 6.7 to 6.9.

Equation 6.7: Two-week tube-component stock
Itube,2wk = 3 830 × 10
Itube,2wk = 38 300 tube components

Equation 6.8: Two-week shovel-head stock
Ihead,2wk = 1 915 × 10
Ihead,2wk = 19 150 blanks

Equation 6.9: Two-week hinge-connector stock
Ihinge,2wk = 1 915 × 10
Ihinge,2wk = 19 150 blanks

For tube storage, each finished shovel requires two tube sections of 150 mm each. The required tube length per shovel and the corresponding daily tube length are calculated in Equations 6.10 and 6.11.

Equation 6.10: Tube length required per shovel
Ltube,shovel = 150 mm + 150 mm
Ltube,shovel = 300 mm = 0.30 m

Equation 6.11: Daily tube length requirement
Ltube,day = 1 819 × 0.30
Ltube,day = 545.7 m/day

Assuming that aluminium tubes are supplied in 6 m stock lengths, the number of stock tubes required per day and for a two-week period is calculated in Equations 6.12 to 6.14.

Equation 6.12: 6 m tubes required per day before scrap
Ntube,before = 545.7 / 6
Ntube,before = 90.95 ≈ 91 tubes/day

Equation 6.13: 6 m tubes required per day after scrap
Ntube,after = 90.95 / (1 - 0.05)
Ntube,after = 95.74 ≈ 96 tubes/day

Equation 6.14: Two-week 6 m tube stock
Itube stock,2wk = 96 × 10
Itube stock,2wk = 960 tubes

Table 6.5: Raw Material Inventory Requirement

| Raw material / component | Daily requirement after scrap | Two-week stock requirement |
| --- | --- | --- |
| 6 m aluminium round tubes | 96 tubes/day | 960 tubes |
| Shovel head blanks | 1 915/day | 19 150 blanks |
| Hinge connector blanks | 1 915/day | 19 150 blanks |

The tube calculation assumes 6 m supply lengths. The assignment brief specifies the tube diameter and handle-section length, but does not explicitly state the supplied tube length; therefore, the 6 m tube length is treated as a design assumption.

## Manufacturing Time and Capacity

The available productive time per day is based on a 9-hour shift less a 30-minute lunch break and two 15-minute tea breaks, as shown in Equation 6.15.

Equation 6.15: Available production time per day
H = 8 × 60
H = 480 min/day

Following the textbook machine-fraction method, the machine or workstation requirement is determined from the total daily processing time divided by the available production time per workstation per day.

The number of required machines or workstations is determined by dividing the total daily processing time by the available production time per workstation per day, as shown in Equation 6.16.

Equation 6.16: Required machines or stations
N = Tday / H

The daily processing time required for any operation is obtained from the product of the daily quantity and the processing time per unit, as shown in Equation 6.17.

Equation 6.17: Daily processing time required
Tday = Q × t

Table 6.6: Production Processing Time Constraints

| Process | Processing time |
| --- | --- |
| Cut handle | 15 s per cut |
| Internal thread on middle section | 20 s per pipe |
| Stamp form end of grip section to narrow | 5 s per pipe |
| Knurling and external thread on grip section | 1 min per pipe |
| Stamp hinge connector | 2 s per part |
| Bend hinge connector | 5 s per part |
| Stamp shovel head | 2 s per part |
| Sharpen shovel head blade | 2 min per shovel |
| Serrate shovel head blade edge | 1 min per shovel |
| Shape/bend shovel head | 2 s per shovel head |
| Rivet hinge connector to shovel head | 5 s per shovel |
| Heat treat shovel head subassembly | Depends on equipment |
| Sandblasting shovel heads | Depends on equipment |
| Hand polish to remove burrs | 1 min per shovel head |
| Powder-coat all parts | Depends on equipment |
| Assemble shovel head and handle section | 30 s per shovel |
| Packaging | Depends on workstation design |

The processing times are taken from the assignment brief.

## Manufacturing Processes and Workstations

Tube Processing

The Tube Processing Department produces the two aluminium handle components required for each folding mini shovel: the middle handle section and the back handle section. The middle handle section is cut to length and internally threaded. The back handle section is cut to length, narrowed at one end, externally threaded, and knurled at the grip end. The DigMore brief specifies aluminium round tube with an outside diameter of 31.70 mm and wall thickness of 3.18 mm, with a 5% scrap allowance for tube cutting.

Figure 6.5: Tube Handle Component Breakdown

Insert labelled Draw.io figure here.

Suggested figure content:

BACK HANDLE SECTION
150 mm aluminium tube
Knurled grip section, tube body, and narrowed threaded end.

MIDDLE HANDLE SECTION
150 mm aluminium tube
Internally threaded end and tube body.

Figure 6.5: Tube Handle Component Breakdown

Process Flow

The tube process follows a split-flow arrangement after cutting. Both handle sections begin as cut tube blanks, but the two parts then follow different processing routes before meeting again at prepared-handle WIP storage.

Raw Tube Staging
↓
Tube Cutting
↓
Component Split
┌───────────────┬─────────────────────┐
↓                ↓
Middle Handle      Back Handle
↓                ↓
Internal Threading   End Narrowing
↓                ↓
↓                External Threading
↓                ↓
↓                Knurling
└───────────────┴─────────────────────┘
↓
Prepared Handle WIP
↓
Final Assembly

Figure 6.6: Tube Processing Process Flow

Insert Draw.io version of the process flow above.

Machine

The selected equipment below is provisional and should be treated as the first design selection for workstation sizing. Each process is linked to a selected machine, a machine specification table, a reference figure, and a workstation layout, following the same structure for every department in this section.

Table 6.9: Tube Processing Equipment Selection

| Operation | Selected equipment | Reason for selection | Source |
| --- | --- | --- | --- |
| Tube cutting | HIPPO CS425CNC-O high-speed aluminium cutting machine | Suitable for aluminium and soft-metal cutting; listed cutting capacity includes round pipe up to Ø150 mm × 6 mm; machine size is 2200 × 1100 × 2200 mm. | hippobender.com |
| Internal threading | SFX-M30R / M6–M30 electric tapping machine | Suitable for aluminium tapping; tapping range includes M6–M30; machine size is listed as 810 × 770 × 220 mm. | brokentapremover.com |
| End narrowing | SG40NC pipe end forming / shrinking machine | Suitable for tube shrinking/reducing; listed maximum tube capacity is Ø40 × 2 mm and machine size is 2000 × 550 × 1300 mm. | liyemachine.en.made-in-china.com |
| External threading | Asada B50-AT pipe threading machine | Listed pipe capacity is 1/2″–2″; automatic die-head version with 750 W motor. | cltoolcentre.com.au |
| Knurling | Round tube embossing / knurling machine | Listed as a round tube embossing/knurling machine with size 1400 × 600 × 1500 mm. | alibaba.com |
| Tube WIP cart | FlexQube tube cart | Listed as a tube cart for transporting long tubes, with dimensions 3080 × 630 mm and load capacity of 2425 lb. | flexqube.com |
| Scrap bin | Industrial steel waste / scrap bin | Local South African suppliers manufacture heavy-duty steel waste containers for scrap metal and industrial offcuts; exact size should be requested from supplier. | macnay.co.za |

Figure 6.7: Tube Cutting Machine Reference
Insert image of HIPPO CS425CNC-O tube cutting machine here.

Figure 6.8: Internal Threading / Tapping Machine Reference
Insert image of SFX-M30R or similar electric tapping machine here.

Figure 6.9: End-Forming / Narrowing Machine Reference
Insert image of SG40NC pipe end forming machine here.

Figure 6.10: External Threading Machine Reference
Insert image of Asada B50-AT pipe threading machine here.

Figure 6.11: Knurling Machine Reference
Insert image of round tube knurling/embossing machine here.

Figure 6.12: Tube Processing Workstation Layout
Insert Draw.io layout here.

Figure 6.13: Tube Cart / Raw Tube Handling Reference
Insert image of tube cart or long-stock trolley here.

Machines

The following tables record provisional machine specifications for layout sizing and capacity justification. Final selection must still be confirmed with suppliers.

Table 6.10: Tube Cutting Machine Specifications

| Equipment specification | Value |
| --- | --- |
| Selected machine | HIPPO CS425CNC-O high-speed aluminium cutting machine |
| Main function | Cutting aluminium and other soft-metal tube/profile material |
| Cutting capacity | Round pipe up to Ø150 mm × 6 mm |
| Cutting accuracy | ±0.05 mm |
| Machine dimensions | 2200 × 1100 × 2200 mm |
| Operator requirement | 1 operator assumed |
| Cost | Supplier quotation required |
| Source | hippobender.com |

Note: The DigMore tube wall thickness is 3.18 mm, while the SG40NC source lists Ø40 × 2 mm. This machine is useful as a reference for layout sizing, but the final selected machine must be confirmed with a supplier for 31.70 mm OD × 3.18 mm wall aluminium tube.

| Table 6.13: External Threading Machine Specifications |  |  |
| --- | --- | --- |
| Equipment specification |  | Value |
| Selected machine |  | Asada B50-AT pipe threading machine |
| Main function |  | External pipe/thread cutting |
| Pipe capacity |  | 1/2″–2″ |
| Spindle speed |  | 40 rpm |
| Motor power |  | 750 W |
| Machine dimensions |  | 640 × 570 × 500 mm |
| Machine weight |  | 71 kg |
| Operator requirement |  | 1 operator assumed |
| Cost |  | Supplier quotation required |
| Source |  | generaltools.com.au |
| Table 6.14: Knurling Machine Specifications |  |
| Equipment specification | Value |
| Selected machine | Round tube embossing / knurling machine |
| Main function | Tube grip pattern forming |
| Machine dimensions | 1400 × 600 × 1500 mm |
| Machine weight | 330 kg |
| Power | 1500 W |
| Required air pressure | 50 litres listed by supplier |
| Operator requirement | 1 operator assumed |
| Cost | Supplier quotation required |
| Source | alibaba.com |

Tube Processing Workstation

The workstation requirement is based on the Section 6.2 capacity analysis and the selected provisional equipment.

| Table 6.15: Tube Processing Workstation Requirements |  |  |  |
| --- | --- | --- | --- |
| Operation | Processing time | Selected equipment | Stations required |
| Tube cutting | 15 s per cut | HIPPO CS425CNC-O cutting machine | 2 |
| Internal threading | 20 s per pipe | SFX-M30R electric tapping machine | 2 |
| End narrowing | 5 s per pipe | SG40NC end forming machine | 1 |
| External threading and knurling | 1 min per pipe | Asada B50-AT + round tube knurling machine | 4 |

Processing times are from the DigMore brief.

Tube Processing Workstation

Figure 6.14: Tube Processing Workstation Layout

Legend:
[RM] Raw material
[M] Machine
[W] WIP storage
[S] Scrap/offcut bin
[OA] Operator aisle

┌────────────────────────────────────────────────────────────┐
│                         [OA] Operator Aisle                │
├──────────────────────┬───────────────────┬────────────────┤
│ [RM] Raw Tube Rack   │ [M] Cutting x 2   │ [S] Scrap Bin  │
│ 6 m tube stock       │                   │                │
├──────────────────────┴─────────┬─────────┴────────────────┤
│                                ↓                           │
│                     [W] Cut Blank Sorting                  │
├───────────────────────────────┬────────────────────────────┤
│ Middle Handle Line            │ Back Handle Line           │
│ [M] Internal Tapping x 2      │ [M] End Narrowing x 1      │
│ [W] Middle Handle WIP         │ [M] External Thread x 4    │
│                               │ [M] Knurling x 4           │
│                               │ [W] Back Handle WIP        │
├───────────────────────────────┴────────────────────────────┤
│                 [W] Prepared Handle WIP to Assembly        │
└────────────────────────────────────────────────────────────┘

Material Handling

Material handling follows a consistent logic across the facility: raw material is staged, processed parts are transferred using carts, WIP-before and WIP-after are clearly shown, and scrap is collected at the process where it is generated.

Table 6.16: Tube Processing Material Handling

| Flow | Handling method | Destination | Figure placeholder |
| --- | --- | --- | --- |
| Raw tube to cutting | Tube rack / tube cart | Tube cutting station | Figure 6.13 |
| Cut blanks to sorting | Small WIP bin/cart | Component split point | Figure 6.14 |
| Middle handle blanks | WIP bin/cart | Internal threading station | Figure 6.14 |
| Back handle blanks | WIP bin/cart | End narrowing station | Figure 6.14 |
| Finished handle parts | Labelled WIP cart | Final assembly | Figure 6.15 |
| Tube scrap/offcuts | Steel scrap bin | Waste handling area | Figure 6.16 |

Figure 6.14: Cut Blank WIP Bin / Sorting Cart
Insert image of small WIP bin or trolley here.

Figure 6.15: Prepared Handle WIP Cart
Insert image of labelled WIP trolley here.

Figure 6.16: Scrap / Offcut Bin
Insert image of industrial steel scrap bin here.

Tube Processing Area Calculation

The final area must be refined after exact supplier equipment is selected. The following calculation gives a defensible first-pass layout allowance.

Equation 6.3: Tube Processing Area
Atube = (Amachines + Araw + AWIP + Ascrap + Aaisle) × 1.10

| Table 6.17: Machine Footprint Calculation |  |  |  |
| --- | --- | --- | --- |
| Equipment | Quantity | Footprint per unit | Total footprint |
| Tube cutting machine | 2 | 2.2 × 1.1 = 2.42 m² | 4.84 m² |
| Internal tapping machine | 2 | 0.81 × 0.77 = 0.62 m² | 1.24 m² |
| End-forming machine | 1 | 2.0 × 0.55 = 1.10 m² | 1.10 m² |
| External threading machine | 4 | 0.64 × 0.57 = 0.36 m² | 1.44 m² |
| Knurling machine | 4 | 1.4 × 0.6 = 0.84 m² | 3.36 m² |
| Total machine footprint |  |  | 11.98 m² |

Machine dimensions used in this table are taken from the selected equipment sources.

| Table 6.18: Tube Processing Area Allowance |  |  |
| --- | --- | --- |
| Area component | Value | Basis |
| Machine footprint | 11.98 m² | From Table 6.17 |
| Raw tube staging | 6.00 m² | Assumed 6 m × 1 m raw tube rack space |
| WIP sorting and prepared handle WIP | 4.00 m² | Assumed WIP bins/carts for split flow |
| Scrap/offcut bin | 3.25 m² | Assumed 2.5 m × 1.3 m scrap bin footprint |
| Operator and trolley movement allowance | 26.00 m² | Design allowance for safe movement between machines |
| Subtotal | 51.23 m² |  |
| 10% expansion allowance | 5.12 m² | DigMore brief requires about 10% expansion space |
| Preliminary tube processing area | 56.35 m² |  |
| Recommended rounded area | 57 m² | Use in Section 6.5 space summary |

The Tube Processing Department should therefore be allocated a preliminary area of approximately 57 m². This area includes the selected machine footprints, raw tube staging, WIP storage, scrap/offcut collection, operator movement, trolley movement, and the 10% future expansion allowance.

Table 6.19: Tube Processing Department Summary

| Item | Final value / decision |
| --- | --- |
| Department input | Aluminium round tube |
| Department output | Prepared back and middle handle sections |
| Main equipment | Cutting, threading, end-forming, knurling |
| Main bottleneck risk | External threading and knurling |
| Scrap source | Tube cutting |
| Material handling | Tube rack, WIP carts, scrap bin |
| Next department | Final assembly |
| Preliminary department area | 57 m² |

## Receiving and Dispatch

Receiving and Dispatch

Storage and Material Handling
Supporting Facilities
Space Requirements

## Space Requirements

The space requirements below are derived from the capacity and demand model (single source of truth workbook), applying the production, storage, personnel and site assumptions set out in the Assumptions and Constraints section. Figures marked as a designer assumption remain to be verified against supplier or civil quotations before final sign-off.

### Production Floor Space

| Area / department | Floor area (m2) | Basis |
| --- | --- | --- |
| Production workstations | 238 | 34 machines x 7 m2 per workstation (designer assumption, incl. operator space) |
| Powder coating (enclosed) | 135 | Supplier-based estimate; conveyorised line, fume extraction, air lock |
| Heat treat and sandblast | 70 | Supplier-based estimate; T6 ageing oven and sandblast booth |

### Storage Space Requirements

Raw material and finished goods quantities are drawn from the demand and inventory model (2-week raw material cover, 4-week finished goods cover). Rack and stacking arrangements are designer assumptions to be verified once racking suppliers are engaged.

| Store | Items | Racks / positions | Floor area (m2) | Basis |
| --- | --- | --- | --- | --- |
| Raw tube | 900 tubes | 5 | 37.5 | Cantilever racking, 200 tubes per rack (designer assumption) |
| Raw sheet | 230 sheets | 2 | 9.4 | Sheet stacks, 150 sheets per stack (designer assumption) |
| Finished goods | 102 pallets | 26 | 46.8 | Palletised, racked, 4 storage levels (designer assumption) |
| WIP staging | 6 units | 2 | 3.6 | Decoupling buffers, approximately one shift |
| Scrap / waste | - | - | 30 | Designer assumption |

Total storage area: 127.3 m2.

### Supporting Facilities Space Requirements

Personnel numbers combine the named roles required by the brief with designer-assumed support staff and the 34 production operators derived 1:1 from the capacity model.

| Category / role | Number | Note |
| --- | --- | --- |
| Senior Plant Manager | 1 | Brief |
| Operations Manager | 1 | Brief |
| Inventory Manager | 1 | Brief |
| Reception | 1 | Brief |
| Purchasing | 1 | Brief |
| Sales | 2 | Brief (assumed count) |
| Finance | 1 | Brief |
| Marketing | 1 | Brief |
| Human Resources | 1 | Brief |
| Plant supervisors | 3 | Designer assumption, approximately one per department |
| Security | 2 | Designer assumption |
| Material handlers / forklift operators | 4 | Designer assumption |
| Equipment-dependent and QA operators | 5 | Heat treat, sandblast, powder coat, inspection (designer assumption) |
| Production operators | 34 | From capacity model, line processes 1:1 |
| TOTAL headcount | 58 |  |

Office and amenity space:

| Area / department | Floor area (m2) | Basis |
| --- | --- | --- |
| Receiving and dispatch | 220 | Docks and staging (designer assumption) |
| Offices | 216 | 24 non-operator staff x 9 m2 per person (designer assumption) |
| Amenities (lockers, restrooms, cafeteria) | 203 | 58 headcount x 3.5 m2 per person (designer assumption) |

Parking and site vehicle provision:

| Item | Number / area | Basis |
| --- | --- | --- |
| Employee bays | 58 bays | 1 bay per on-site employee |
| Visitor and collection bays | 8 bays | Designer assumption |
| Total car bays | 66 bays, 1 848 m2 | 28 m2 per bay including aisle (designer assumption) |
| Truck fleet bays | 5 bays, 350 m2 | Own fleet, 70 m2 per bay including manoeuvring (designer assumption) |
| Total vehicle area | 2 198 m2 |  |

### Total Building and Site Footprint

| Item | Floor area (m2) |
| --- | --- |
| Subtotal (functional floor area) | 1 209.3 |
| Circulation and aisle allowance (25%) | 302.3 |
| Building subtotal | 1 511.6 |
| Future expansion provision (approximately 10%) | 151 |
| TOTAL BUILDING FOOTPRINT | 1 663 |
| Parking and truck fleet (site) | 2 198 |
| INDICATIVE SITE FOOTPRINT | 3 861 |

All figures above are traceable to the capacity and space workbook and will be refined once supplier quotations are obtained for enclosed process areas, racking and paving rates.

# Design Alternatives

Three alternative block layouts were developed from the activity relationships implied by the process flow (Section 5) and the department groupings used in the space requirements above (Section 6.6). Each alternative arranges the same set of departments (receiving, raw material storage, tube processing, sheet and hinge processing, heat treat and surface preparation, the enclosed powder coating area, assembly and packaging, finished goods storage, dispatch, and support facilities) into a different flow pattern, so that the choice between them turns on flow logic, hazard separation and expansion capacity rather than on department content.

## Alternative 1: Linear Spine

A single-direction flow from receiving to dispatch, with departments laid out left to right along one production spine. This gives the most direct, easiest-to-follow material flow and the simplest construction sequence. Its main weakness is that the enclosed powder-coating area sits mid-line, which lengthens the fume-extraction duct run and brings coating traffic closer to pedestrian routes than is ideal.

## Alternative 2: U-Shape Return

Receiving and dispatch share one face of the building, with the process flow wrapping around in a U so that both external-facing activities are on the same side. This is the most compact option and suits efficient internal material handling, but it crowds the powder-coating enclosure against active production areas and leaves the least room for the required 10% expansion margin within a tight footprint.

## Alternative 3: Parallel-Branch with Finishing Spine

Tube processing and sheet/hinge processing run as two parallel halls, each feeding into a perpendicular finishing, assembly and packaging spine. Powder coating sits as a separated block off the spine with its own dedicated extraction, giving the strongest hazard separation of the three options and a clear direction (an additional feeder cell) in which to apply the 10% expansion allowance. The trade-off is a somewhat higher build complexity and more pedestrian crossings where the two feeder halls meet the spine.

## Evaluation Criteria and Weighting

Each alternative was scored from 1 (poor) to 5 (excellent) against seven criteria drawn directly from the design requirements and constraints, weighted by their relative importance to the brief. The weighted score for each alternative is the sum of its criterion scores multiplied by the criterion weight.

| Criterion | Weight |
| --- | --- |
| Meets demand of 400 000 shovels/yr | 0.20 |
| Logical material flow (receiving to dispatch) | 0.20 |
| Enclosed powder coating and hazard separation | 0.15 |
| Efficient storage and material handling | 0.15 |
| Pedestrian / vehicle safety and site access | 0.10 |
| Approximately 10% expansion provision | 0.10 |
| Ease of construction / implementation | 0.10 |

## Weighted Evaluation

| Criterion | Weight | Alt 1: Linear Spine | Alt 2: U-Shape | Alt 3: Parallel-Branch |
| --- | --- | --- | --- | --- |
| Meets demand of 400 000/yr | 0.20 | 5 | 5 | 5 |
| Logical material flow | 0.20 | 5 | 3 | 4 |
| Enclosed powder coating / hazard separation | 0.15 | 3 | 2 | 5 |
| Efficient storage and material handling | 0.15 | 3 | 4 | 3 |
| Pedestrian / vehicle safety | 0.10 | 4 | 3 | 4 |
| Approximately 10% expansion provision | 0.10 | 3 | 2 | 5 |
| Ease of construction | 0.10 | 5 | 4 | 3 |
| Weighted total | 1.00 | 4.10 | 3.40 | 4.20 |

Alternative 3 (Parallel-Branch with Finishing Spine) scores highest overall, driven by its stronger hazard separation and expansion margin, both of which are explicit brief requirements (fully enclosed powder coating; approximately 10% expansion). Alternative 1 remains competitive on flow simplicity and ease of construction, while Alternative 2's compactness is offset by its weaker expansion margin. Alternative 3 is therefore carried forward as the basis for the Master Facility Plan.

All figures in this section are derived from the capacity and space workbook (single source of truth). The scoring reflects a first-pass designer judgement against the stated criteria and should be revisited once the Master Facility Plan is drawn in detail.

[Figure pending: block layout sketches for Alternatives 1, 2 and 3 - see Alternatives.drawio.pdf for prior exploratory layouts, to be finalised against the department areas in Section 6.6]