DigMore Folding Mini Shovel: Dedicated Manufacturing Facility Design

Sections 1 to 4 (v2): Background, Objectives, Requirements, and Product and Process Understanding

# 1   Background

DigMore Co. is an established shovel manufacturer with a broad local and international customer base. Management is extending its range with an all-purpose folding mini shovel for camping and outdoor use. The shovel is assembled from three aluminium parts, two handle sections and a riveted shovel-head subassembly, finished in a black powder coating and packed by hand.

Demand for the new product is projected at 400 000 units per year. The existing plant, configured for DigMore's current shovel ranges, does not have the spare capacity, floor area or process route to absorb this volume without disrupting current production. Management therefore requires a new dedicated facility, with access from public roads and approximately 10 percent of space reserved for five-year growth, to support strategic decisions on the size, layout and investment of the new plant.

This report applies a systematic engineering design approach to that facility. It establishes the design requirements and the demand basis, analyses the product and its manufacturing processes, sizes the capacity, machinery, labour, storage and space, evaluates alternative layouts, and delivers a justified Master Facility Plan.

# 2   Design Objectives

A well-resolved facility lets DigMore meet the forecast demand reliably and safely while controlling cost and enabling growth. The aim and objectives below direct the design.

## 2.1   Aim

The aim of this project is to develop, through a systematic engineering design approach, a justified Master Facility Plan for a new dedicated DigMore facility that meets the forecast demand for the folding mini shovel, provides safe and efficient material and personnel flow, satisfies the storage, receiving and dispatch policies, and allows for future expansion.

## 2.2   Objectives

Production and capacity

- Translate the annual demand into daily output and takt time, and size the machines and operators per process, allowing for scrap.

- Identify bottlenecks and the process changes needed to meet demand.

Layout and flow

- Define the departments and workstations, and design material and personnel flow from receiving to dispatch with minimal travel and back-tracking.

Storage and inventory

- Size raw material (two weeks), work-in-progress and finished goods (four weeks) storage in three dimensions, all indoors.

Receiving and dispatch

- Provide supplier receiving, on-site small-order collection, large-order dispatch and company truck-fleet space, and define the small and large order quantities.

Safety and quality

- Enclose powder coating, separate hazardous operations and pedestrian and vehicle flow, and embed the process flow charts and the final visual inspection.

Support facilities and expansion

- Provide offices, amenities, parking, security, waste handling and utilities sized to personnel, and reserve about 10 percent of space for expansion.

Master Facility Plan

- Develop and evaluate at least three layout alternatives against defensible criteria and deliver a justified Master Facility Plan.

## 2.3   Scope and exclusions

The scope is the conceptual design of the DigMore mini shovel facility: demand and capacity analysis, product and process understanding, raw material, work-in-progress and finished goods storage, receiving, collection and dispatch, support facilities, layout alternatives and the final Master Facility Plan. Detailed building, civil, heating and ventilation, and electrical design, full financial feasibility, and final supplier procurement are excluded except where an assumption is needed for space planning. The design is a conceptual front-end plan, not a construction specification.

# 3   Design Requirements, Assumptions and Constraints

## 3.1   Design requirements

Production and capacity. The facility shall:

- produce 400 000 folding mini shovels per year, equal to 1 819 per day, within one 9-hour shift of 480 effective minutes;

- account for 5 percent scrap at tube cutting and at sheet stamping;

- organise machines and operators into workstations that meet a takt of about 15.84 s per shovel.

Storage and inventory. The facility shall:

- store two weeks of raw materials and four weeks of finished goods, all indoors;

- provide work-in-progress staging at the process decoupling points.

Receiving, collection and dispatch. The facility shall:

- receive supplier vehicles to offload raw materials and bought-in components;

- provide on-site small-order customer collection and large-order dispatch on the company truck fleet, during weekday working hours, with on-site space for the fleet.

Safety and environment. The facility shall:

- house powder coating in a separate, fully enclosed and extracted area;

- contain and ventilate heat treatment and sandblasting, separate pedestrian and vehicle movement, and provide segregated waste handling.

Quality. The facility shall:

- embed the process flow charts in the quality management system and include a final visual inspection at the hand-packaging station.

Support facilities and site. The facility shall:

- provide offices for the named management and administrative roles and the supervisors, with restrooms, lockers and a cafeteria for all personnel;

- provide parking, security with controlled access from public roads, utilities, and approximately 10 percent of space reserved for future expansion.

## 3.2   Closeness (adjacency) requirements

The following relationships guide the activity relationship analysis and layout development in the later sections.

- Receiving close to the raw material store; the raw material store close to tube cutting and sheet stamping.

- Cutting and stamping close to their respective work-in-progress buffers.

- Powder coating enclosed and separated from assembly, packaging and finished goods.

- Heat treatment and sandblasting separated and ventilated, away from assembly and packaging.

- Final assembly close to packaging; packaging close to finished goods.

- Finished goods close to dispatch and the collection bay; scrap and waste storage accessible for external removal without crossing production.

## 3.3   Production and demand basis

The facility is sized for 400 000 shovels per year over 220 workdays. The daily requirement and takt time follow:

Daily demand = 400 000 / 220 = 1 818.18, rounded up to 1 819 shovels per day.

Effective time = 540 min shift - 60 min breaks (30 + 15 + 15) = 480 min per day.

Takt time = 480 / 1 819 = 0.264 min, which is about 15.84 s per shovel.

Scrap-adjusted input at cutting and stamping = 1 819 / 0.95, which is about 1 915 units per day.

Per-process machine and labour counts, bottlenecks and recommended machinery follow in the capacity analysis.

## 3.4   Assumptions

Table 1 lists the designer assumptions made where the brief leaves a value open; each is confirmed during detailed design.

| Assumption | Value or basis | Verification required |
| --- | --- | --- |
| Small order | Up to one pallet, collected on site by the customer; unit count set from the packaging load | Confirm against order data |
| Large order | More than one pallet, palletised and dispatched on the company fleet | Confirm against order data |
| WIP policy | Buffer of about one shift at decoupling points (before heat treat, sandblast, powder coat, assembly and packaging) | Refine in line balancing |
| Delivery / dispatch frequency | Scheduled within the two-week buffer; weekday working hours only | Confirm with supplier and logistics |
| Material handling | Forklifts and pallet trucks for pallets; trolleys or conveyors between stations | Confirm against loads and flow |
| Aisle widths | About 3.0 to 3.7 m main (forklift); about 1.5 m pedestrian | Set from final equipment |
| Packaging unit load | Bag per shovel, then boxes, then pallets; units per box and boxes per pallet to be set | Confirm box and pallet sizes |
| Office and supervisor numbers | One per named role; about one supervisor per production department or shift | Confirm headcount with management |
| Parking basis | One bay per on-site employee on shift, plus visitor and collection bays | Finalise once labour is known |
| Heat treat, sandblast, powder coat sizing | Enclosed, extracted equipment sized to throughput; cycle times equipment-dependent | Supplier-based estimate required |

Table 1: Key designer assumptions.

## 3.5   Constraints

The following are fixed by the 2026 brief and are not at the designer's discretion.

- Demand 400 000 shovels per year; 220 workdays; one 9-hour shift, 5 days per week, with 30 + 15 + 15 minutes of breaks (480 effective minutes per day).

- 5 percent scrap at tube cutting and sheet stamping only; no other scrap.

- Two weeks of raw material and four weeks of finished goods stock; all materials stored indoors.

- Powder coating in a separate, fully enclosed area; all packaging by hand with a final visual check.

- Powder coating and sandblasting have a negligible effect on product mass.

- Deliveries and collections during weekday working hours only; large orders on the company truck fleet; access from public roads.

- Approximately 10 percent of space reserved for five-year expansion.

- Raw materials: aluminium round tube OD 31.70 mm, wall 3.18 mm, and sheet 2.5 mm thick (Aluminium Trading).

- Process flow charts must form part of the quality management system; report body limited to 40 pages; Master Facility Plan submitted as source and PDF.

# 4   Product and Process Understanding

## 4.1   Product structure and bill of materials

The shovel is built from two manufactured handle sections and a shovel-head subassembly, which are joined and then packed. Table 2 gives the product structure and Table 3 the preliminary bill of materials; the rivet quantity and packaging load remain designer assumptions to be confirmed.

| Level | Item | Source or type |
| --- | --- | --- |
| 0 | Finished folding mini shovel | Assembled and packed |
| 1 | Back handle section (grip) | From aluminium round tube |
| 1 | Middle handle section | From aluminium round tube |
| 1 | Shovel-head subassembly | Subassembly |
| 2 | Shovel head | From aluminium sheet |
| 2 | Hinge connector | From aluminium sheet |
| 2 | Rivet | Bought-in |
| 1 | Packaging (bag, box) | Bought-in |

Table 2: Product structure.

| Component | Status | Qty/shovel | Main process | Notes |
| --- | --- | --- | --- | --- |
| Back handle (grip) | Made (tube) | 1 | Cut, narrow end, external thread, knurl | Tube OD 31.70, wall 3.18 mm |
| Middle handle | Made (tube) | 1 | Cut, internal thread, hinge to head | Same tube |
| Shovel head | Made (sheet) | 1 | Stamp, bend, sharpen, serrate | 2.5 mm sheet; 150 x 120 mm |
| Hinge connector | Made (sheet) | 1 | Stamp, bend | 2.5 mm sheet |
| Rivet | Bought-in | 1 (maybe 2; verify) | Rivet hinge to head | Quantity is an assumption |
| Bag + box | Bought-in | 1 bag; box per load | Hand packaging | Units per box to be set |

Table 3: Preliminary bill of materials.

## 4.2   Component descriptions

Back handle (grip) section

A 150 mm length of round aluminium tube (OD 31.70 mm, wall 3.18 mm). One end is stamped to narrow it and an external thread is cut there; the other end is knurled into a grip. Operations and brief times: cut handle 15 s per cut, stamp and form the narrowed end 5 s per pipe, and knurl with external thread 1 min per pipe. One per shovel.

Middle handle section

A 150 mm length of the same tube with an internal thread cut at one end to receive the grip; the other end hinges to the shovel head with a rivet, giving the folding action. Operation: internal thread 20 s per pipe. One per shovel.

Shovel head and hinge connector

Both are stamped from 2.5 mm aluminium sheet. The head is stamped (2 s), shaped and bent (2 s), sharpened (2 min) and serrated (1 min); the finished head is 150 mm by 120 mm. The hinge connector is stamped (2 s) and bent (5 s), then riveted to the head (5 s) to form the shovel-head subassembly, which is heat treated, sandblasted and hand polished (1 min per head) before powder coating.

## 4.3   Raw materials and bought-in components

| Item | Type | Specification or source | Used for | Notes |
| --- | --- | --- | --- | --- |
| Aluminium round tube | Raw material | OD 31.70 mm, wall 3.18 mm; Aluminium Trading | Handle sections | Two 150 mm sections per shovel |
| Aluminium sheet / coil | Raw material | 2.5 mm thick; Aluminium Trading | Head and hinge connector | Alternative supplier may be proposed |
| Rivets | Bought-in | To be confirmed | Rivet hinge to head | 1 per shovel, possibly 2 |
| Packaging bags | Bought-in | From an outside supplier | Bagging each shovel | 1 per shovel |
| Packaging boxes | Bought-in | To be confirmed | Boxing bagged shovels | Units per box is an assumption |
| Black powder-coat material | Bought-in | To be confirmed (supplier estimate) | Finish on all parts | Quantity from coated area |

Table 4: Raw materials and bought-in components.

## 4.4   Overall process flow

Raw materials are received, inspected and stored indoors, then split into a tube stream and a sheet stream that run in parallel. The tube stream cuts handle blanks (5 percent scrap) and branches into the back-handle and middle-handle paths. The sheet stream branches into the shovel-head and hinge-connector paths (5 percent scrap at stamping). The head and hinge converge at riveting to form the shovel-head subassembly, which is heat treated, sandblasted and polished. All coated parts then converge at the enclosed powder-coating area, after which the shovel is assembled, hand packed with a final visual inspection, stored, and either collected by the customer (small orders) or dispatched on the company fleet (large orders). Scrap from cutting and stamping exits to waste storage. The complete flow, with the parallel streams, scrap exits, work-in-progress points and the enclosed powder-coating area, is shown in Figure 1.

Figure 1: DigMore folding mini shovel process flow diagram.

## 4.5   Process route sheet

Processing times are taken directly from the brief; equipment- or workstation-dependent times are shown as to be determined. The 5 percent scrap points (tube cutting and sheet stamping) are flagged in the note column.

| Step | Component | Operation | Input | Output | Time (brief) | Equipment / workstation | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Raw material | Receiving and inspection | Tube, sheet, bought-in items | Inspected material in store | Not specified | Receiving dock / inspection | Incoming quality check |
| 20 | Handle | Cut handle | Aluminium round tube | 150 mm sections (back, middle) | 15 s per cut | Tube cutting / saw | 5% scrap point |
| 30 | Middle handle | Internal thread | 150 mm tube section | Threaded middle section | 20 s per pipe | Threading machine | Receives grip |
| 40 | Back handle | Stamp / form narrow end | 150 mm tube section | Narrowed-end tube | 5 s per pipe | Stamping / forming | Prepares for thread |
| 50 | Back handle | Knurl + external thread | Narrowed-end tube | Threaded, knurled grip | 1 min per pipe | Knurl / thread machine | Grip and male thread |
| 60 | Shovel head | Stamp shovel head | Aluminium sheet | Head blank | 2 s per part | Stamping press | 5% scrap point |
| 70 | Shovel head | Shape / bend head | Head blank | Formed head | 2 s per head | Press / bending | Forms to shape |
| 80 | Shovel head | Sharpen blade | Formed head | Sharpened head | 2 min per shovel | Sharpening station | Edge prep |
| 90 | Shovel head | Serrate edge | Sharpened head | Serrated head | 1 min per shovel | Serrating station | Serrated edge |
| 100 | Hinge connector | Stamp hinge connector | Aluminium sheet | Connector blank | 2 s per part | Stamping press | 5% scrap point |
| 110 | Hinge connector | Bend hinge connector | Connector blank | Formed connector | 5 s per part | Bending station | Bent to shape |
| 120 | Head subassembly | Rivet hinge to head | Head + connector + rivet | Shovel-head subassembly | 5 s per shovel | Riveting station | Hinged joint |
| 130 | Head subassembly | Heat treat | Subassembly | Heat-treated subassembly | To be determined | Heat-treatment oven | Equipment-dependent |
| 140 | Head subassembly | Sandblast | Subassembly | Sandblasted subassembly | To be determined | Enclosed blast booth | Extraction required |
| 150 | Head subassembly | Hand polish / deburr | Subassembly | Deburred subassembly | 1 min per head | Polishing station | Surface finish |
| 160 | All parts | Powder-coat (black) | Handles + subassembly | Coated parts | To be determined | Enclosed powder-coat line | Fully enclosed; safety |
| 170 | Finished shovel | Final assembly | Coated parts + rivet | Assembled shovel | 30 s per shovel | Assembly station | Grip screws in; hinged head |
| 180 | Finished shovel | Hand packaging + visual check | Shovel + bag + box | Packaged shovel | To be determined | Packaging station | Final quality check |
| 190 | Finished goods | Finished goods storage | Packaged shovels | Stored stock | Not applicable | Finished goods store | 4 weeks stock |
| 200 | Finished goods | Collection / dispatch | Packaged shovels | Collected / dispatched order | Not applicable | Collection bay / dock | Small = collect; large = fleet |

Table 5: Preliminary process route sheet (times as stated in the 2026 brief).