Submitted in pursuit of the degree

BACHELOR OF ENGINEERING

In

INDUSTRIAL ENGINEERING

# INGB417 - FACILITIES DESIGN

GA Exam Assignment 2

Student: Nthabiseng Maoka

Student Number: 30224926

Lecturer: Mrs. Willemien Kruger

Due date: 03 July 2026

North-West University: Potchefstroom Campus

# Executive Summary

The report presents the design of a new dedicated manufacturing facility for DigMore Co.’s folding mini shovel product line. The project involved identifying the machinery, equipment, labour requirements, workstations, storage areas and material-handling methods needed to support the full production process. The design considered the movement of aluminium tube and sheet-metal components from receiving and raw material storage through manufacturing, finishing, assembly, manual packaging and dispatch. Alternative facility layouts were developed and assessed against the design objectives and requirements, with the preferred option refined into the final master facility plan. The business case for the new facility was also examined by considering the expected costs, main risks, mitigation actions and implementation schedule.

# ECSA Graduate Attribute 3 Alignment

ECSA Graduate Attribute 3: Design / synthesis of solutions

According to the Engineering Council of South Africa, Graduate Attribute 3 requires students to:

Demonstrate competence to perform creative, procedural and non-procedural design and synthesis of components, systems, works, products or processes to meet desired needs normally within applicable standards, codes of practice and legislation. (Engineering Council of South Africa, 2023).

The table below illustrates how ECSA GA3 was addressed in this facilities design assignment.

ECSA Graduate Attribute 3 alignment

| GA3 requirement | How it was addressed in the INGB417 2026 examination assignment |
| --- | --- |
| Conducting an investigation of a complex engineering problem | The assignment investigated a quality assurance problem at Tangra Inc., in which defective tangram puzzle pieces compromised the customers’ ability to assemble figurines correctly. The investigation focused on whether the company could identify defective parts efficiently while still assembling enough complete figurines to meet demand. |
| Use of research methods and research-based knowledge | Relevant quality assurance and quality management literature was used to support the investigation. The report applied quality concepts from both the customer and organisational perspectives, including defect identification, process performance, statistical process control, and quality improvement. |
| Design and execution of an experiment | The assignment required an experiment in the Learning Factory. Two quality testing processes were followed. Process 1 involved sampling five pieces, testing defects, removing defective parts and assembling the figurine. Process 2 involved picking parts from sorted bins, assembling a figurine and discarding the entire semi-assembled item if a defect was found. |
| Data collection | Data was collected during the allocated 20-minute testing period for each process. The collected data included defects, completed figurines, defective pieces, discarded assemblies and any other relevant observations needed to compare the two processes. |

- Table of Contents

Executive Summary	1

ECSA Graduate Attribute 3 Alignment	1

List of Tables	1

List of Figures	1

1.	Background	2

2.	Design objectives	2

2.1.	Aim	2

2.2.	Objectives	2

3.	Design requirements	3

4. Assumptions and Constraints	5

4.1. Assumptions	5

4.2. Constraints	6

4.3. Raw materials and processing times	6

4.	Process Flow	7

5.	Detailed Design Specifications	7

5.1.	Raw Material Requirements	7

References	9

# List of Tables

Table 1:Assumptions used in the Facility Design	5

Table 2:Process time constraints	6

# List of Figures

# Background

DigMore Co. is an established shovel manufacturer with a broad customer base in the camping and outdoor market. The company has decided to expand its product range by introducing an all-purpose folding mini shovel designed for outdoor use. The product is manufactured from aluminium round tubes and sheets, comprising three main sub-assemblies: a back handle section (narrowed with a knurled grip and external thread), a middle handle section (with an internal thread and riveted hinge connection), a stamped shovel head, and a stamped hinge connector.

Due to the projected surge in demand for this new product, the existing manufacturing facility lacks the capacity to accommodate the new product line without compromising current operations. Consequently, management has decided to establish a dedicated, separate facility for the folding mini shovel.

The new facility must accommodate receiving docks, indoor raw-material storage, production operations, work-in-progress (WIP) staging, finished goods storage, customer collections, and dispatch operations. Additionally, the design must incorporate a fully enclosed powder-coating department to manage chemical fumes and safety hazards, and provision must be made for a 10% space buffer for future expansion. This report applies a systematic engineering design process (Muther’s Systematic Layout Planning framework) to define facility requirements, evaluate layout alternatives, and present a final Master Facility Plan (MFP) for strategic management approval.

Design objectives

## Aim

Design a manufacturing facility for DigMore Co. that meets the projected demand, provides efficient receiving and dispatch operations, ensures safe and logical material flow, and allows for future expansion.

## Objectives

The following objectives guide the development of the new facility:

- Production and Capacity

To design a facility that can meet the projected demand by providing the required machinery, labour, workstations and production capacity for each manufacturing process.

- Site and Access

To develop a site layout that provides safe access from public roads for supplier deliveries, customer collections, employee movement and DigMore Co.’s delivery vehicles.

- Layout and Material Flow

To develop a layout that supports a clear flow of materials from receiving and raw-material storage through production, work-in-progress staging, assembly, packaging, finished-goods storage and dispatch.

- Equipment and Workstations

Select machinery, equipment and workstation designs that support the required operations, including tube cutting, threading, stamping, bending, blade finishing, riveting, heat treatment, sandblasting, polishing, powder coating, assembly and packaging.

- Storage and Material Handling

Provide indoor storage for raw materials, work-in-progress and finished goods, supported by suitable material-handling methods between receiving, production, storage and dispatch areas.

- Safety and Support Facilities

Incorporate the required safety provisions and environmental regulations, including a fully enclosed powder-coating area, and provide offices, amenities, parking, utilities and other support facilities for management, administrative staff and operators.

# Design requirements

The following design requirements, categorised under their relevant headings, have been established to guide the design of DigMore Co.’s new folding mini shovel manufacturing facility.

Production and Capacity

The facility must:

- Produce the required annual demand for the folding mini shovel product line.

- Operate according to the specified single-shift structure, with breaks considered when determining productive time.

- Allow for scrap at the tube cutting and sheet stamping processes only.

- Include manual packaging workstations, as packaging also serves as the final visual inspection point before shipping.

Manufacturing Processes

The facility must:

- Accommodate the complete manufacturing process, including tube cutting, threading, grip forming, knurling, sheet stamping, bending, sharpening, serrating, riveting, heat treatment, sandblasting, polishing, powder coating, assembly and packaging.

- Provide suitable machinery, workstations, operator space and material-handling space for each operation.

- Include work-in-progress (WIP) staging areas between operations where required.

Site, Access and Vehicle Movement

The facility must:

- Be accessible from public roads for supplier deliveries, customer collections, employees and delivery trucks.

- Provide space for delivery vehicles to enter, offload, load and exit safely.

- Include a designated collection point for small customer orders.

- Provide space for DigMore Co.’s truck fleet.

- Include parking, security control and access control for employees, customers, visitors and goods vehicles.

Figure 2:loading bay

Figure 1:vehicle parking

Layout and Flow

The facility must:

- Provide a clear path from receiving to raw-material storage, production, WIP staging, assembly, packaging, finished-goods storage and dispatch.

- Provide clear movement paths for operators, materials and handling equipment.

- Position packaging after assembly and before finished-goods storage.

- Position finished-goods storage close to dispatch.

Storage and Material Handling

The facility must:

- Store all raw materials, components, WIP and finished goods inside the building.

- Provide storage for aluminium round tubes, aluminium sheets or coils, bought-in packaging materials and completed shovels.

- Provide WIP storage between production stages where required by the selected production design.

- Include staging areas at relevant manufacturing and assembly workstations.

- Use material-handling equipment, unit loads and aisle widths suited to the selected storage and movement requirements.

Receiving and Dispatch

The facility must:

- Include a designated receiving area for supplier deliveries during working hours.

- Provide space for offloading, receiving inspection and transfer of raw materials to storage or production.

- Include a dispatch area for large orders transported by DigMore Co.’s delivery trucks.

- Provide a controlled collection point for small customer orders.

- Include staging space for completed orders before loading or collection.

- Ensure that receiving and dispatch activities do not obstruct internal production movement.

Safety, Environmental and Waste Management

The facility must:

- Place powder coating in a separate, fully enclosed area because of fume-related safety concerns.

- Provide safe separation for cutting, stamping, heat treatment, sandblasting, sharpening, serrating and powder coating where required.

- Provide clear operator pathways around workstations and material-handling routes.

- Include scrap and waste areas for material removed during tube cutting and sheet stamping.

- Position waste collection points so that scrap can be removed without interrupting production flow.

Supporting Facilities

The facility must:

- Provide office space for management, administrative staff, reception, purchasing, sales, finance, marketing, human resources and plant supervisors.

- Provide restrooms, lockers and cafeteria access for all employees.

- Provide parking for management, administrative staff, security personnel, operators, customers and company vehicles.

- Include water, electricity, plumbing, lighting and ventilation requirements.

- Include security facilities to control the movement of people, materials and vehicles.

Specific Layout Implications

The facility must:

- Locate raw-material storage close to the first production operations.

- Keep tube-processing and sheet-metal-processing flows clear before they join at subassembly and assembly stages.

- Place WIP staging areas between related operations without blocking movement paths.

- Separate the enclosed powder-coating area from general production movement.

- Locate packaging between assembly and finished-goods storage.

- Locate finished-goods storage close to dispatch.

- Arrange receiving, customer collection and dispatch so that external vehicle movement does not interfere with internal production flow.

# 4. Assumptions and Constraints

This section summarises the key assumptions, constraints, and material specifications underlying the facility design. Section 4.1 covers the assumptions used in the report, Section 4.2 outlines the design constraints, and Section 4.3 presents the raw materials and processing times for the folding mini shovel.

## 4.1. Assumptions

Table 1:Assumptions used in the Facility Design

| Assumption | Details |
| --- | --- |
| Annual demand | The facility is designed to produce 400 000 folding mini shovels per year. |
| Operating schedule | One 9-hour shift per day, five days per week, for 220 working days per year. A Saturday shift may be used, but only as a flexible option for adjusting capacity when needed. |
| Productivity assumptions | Machine setup times, changeover times, and cleaning times are assumed to be negligible for the purposes of capacity calculations, unless stated otherwise. |
| Productive time per shift | Operators receive a 30-minute lunch break and two 15-minute tea breaks per shift, resulting in 480 minutes (8 hours) of productive time per day. A Saturday shift is possible but should only be considered to provide flexibility in capacity. |
| Work-in-progress stock | WIP levels are determined by the selected production flow and are provided only where staging is required between operations. |
| Packaging | Packaging materials (small bags and boxes) are sourced from an outside supplier. |

## 4.2. Constraints

The final facility design must satisfy the following constraints:

- The facility must be dedicated to the folding mini shovel product line.

- A 5% scrap allowance applies only to the tube cutting and sheet stamping processes.

- All raw materials, components, work-in-progress and finished goods must be stored inside the facility.

- The standard aisle size is 1.5 m wide.

- Operators and personnel are restricted to designated safe movement areas where required for safety and operational efficiency.

- Powder coating must be placed in a separate, fully enclosed area within the building because of fume-related safety concerns.

- The facility must provide access from public roads for supplier deliveries, customer collections, employees and dispatch vehicles.

- Space must be provided on site for DigMore Co.’s own delivery truck fleet.

## 4.3. Raw materials and processing times

The raw materials used to produce the folding mini shovel are supplied in standard lengths and sheet forms. These materials are used to manufacture the handle sections, shovel head and related components. The following materials are required:

- Aluminium round tubes: (31.70 mm outer diameter × 3.18 mm wall thickness × 6000 mm) used to manufacture the back handle section and middle handle section.

- Aluminium sheets: (2.5 mm thickness) used to manufacture the shovel head and hinge connector components.

- Rivets: used to assemble the hinge connector to the shovel head and support the folding mechanism.

- Packaging materials: (plastic bags and carton boxes) sourced externally for final product packaging and distribution.

Processing times

Table 2:Process time constraints

| Production Step | Estimated Processing Time |
| --- | --- |
| Cut handle | 15 s per cut; number of pipes per cut depends on chosen equipment |
| Internal thread on the middle section | 20 s per pipe |
| Stamp form the end of the grip section to narrow | 5 s per pipe |
| Knurling and external thread on the grip section | 1 min per pipe |
| Stamp hinge connector | 2 s per part |
| Bend hinge connector | 5 s per part |
| Stamp shovel head | 2 s per part |
| Sharpen the shovel head blade | 2 min per shovel |
| Serrate shovel head blade edge | 1 min per shovel |
| Shape/bend the shovel head | 2 s per shovel head |
| Rivet hinge connector to shovel head | 5 s per shovel |
| Heat treat shovel head subassembly | Depends on equipment |
| Sandblasting shovel heads | Depends on equipment |
| Hand polish to remove burrs | 1 min per shovel head |
| Powder-coat all parts, black colour | Depends on equipment |
| Assemble the shovel head and handle section | 30 s per shovel |
| Packaging, first into small bags bought from an outside supplier, then into boxes | Depends on workstation design |

# Process Flow

The process flow chart in Figure 3 shows the movement of materials through the DigMore folding mini shovel facility, from receiving and raw-material storage to production, assembly, packaging, finished-goods storage and dispatch. The flowchart identifies the main operations, WIP buffers, decision points, scrap points and material entry and exit points used to guide the facility layout.

The flow separates the aluminium round tube components from the sheet-metal components during the early production stages. These streams later join at the subassembly and final assembly stages before moving through powder coating, packaging and finished-goods storage. This flow logic is used to position related departments close together, provide WIP staging where needed, and separate receiving, dispatch and customer collection activities from internal production movement.

# Detailed Design Specifications

This section presents the calculations and design specifications used to determine the capacity, material requirements, workstation design, and space requirements for the proposed DigMore Co. folding mini shovel manufacturing facility. The purpose of this section is to translate the design requirements into measurable engineering values that inform the final facility layout.

## Raw Material Requirements

# References